UNITED STATES MARINE CORPS

MARINE CORPS DETACHMENT

1273 IOWA AVE

FORT LEONARD WOOD, MISSOURI 65473-8963

E45C06-5

26/7/30

STUDENT OUTLINE

CONTAINER HANDLER OPERATION

LEARNING OBJECTIVE(S)

a.  TERMINAL LEARNING OBJECTIVE(S)

(1) 	Provided with a Rough Terrain Container Handler (RTCH), a Top Handler, an ISO Container, a training area, and two ground guides, without the aid of references, operate the container handler in accordance with TM-3930-14/1.  (1345.2.12)

b.	ENABLING LEARNING OBJECTIVE(S)

(1) Given a list of Rough Terrain Container Handler operating characteristics and a list of container handler operating principles, without the aid of notes or references, match each operating characteristic to its operating principle in accordance with TM-3930-14/1 and TM-08637A-14/1.  (1345.2.12a)

(2) 	Provided with a Rough Terrain Container Handler, lubricants, a training area, and necessary tools, with the aid of references, perform, before operation checks, in accordance with TM-3930-14/1 and TM-08637A-14/1.(1345.2.12b)

(3) Provided with a Rough Terrain Container Handler, a training area, with the aid of references, perform basic container handler operations, in accordance with TM-3930-14/1 and TM-08637A-14/1. (1345.2.12b)

(4) Provided with a Rough Terrain Container Handler, lubricants, a training area, and necessary tools, with the aid of references, perform during operation checks, in accordance with TM-3930-14/1 and TM-08637A-14/1. (1345.2.12b)

(5) Provided with a Rough Terrain Container Handler, lubricants, a training area, and necessary tools, with the aid of references, perform after operation checks, in accordance with TM-3930-14/1 and TM-08637A-14/1. (1345.2.12b)

OUTLINE:

1.	CONTAINERIZED CARGO HANDLING

a.  The concept of "break-bulk" cargo is being phased out in favor of "containerized" cargo.  "Break-bulk" cargo handling is the method employed since the earliest days of shipping.  A bulk of cargo is broken down into groups that can be handled by the equipment available.  Although that method has worked for many years, containerization is the cargo handling method for the future.

b.  Containerization within the material handling industry is a method by which a quantity of cargo is stowed into standard sized metal containers prior to being shipped to the intended destination.  These containers have internally constructed lifting points called "CORNER CASTINGS".

c.  Containerization is the method by which the shipping industry is being standardized.  Standardization of dimensions and weight capacities has spread across the world.  Cargo containers are available in various sizes.  The most common sizes are:

(1) 	8' wide X 8' high X 20' long (ISO)

(2) 	8' wide X 8' high X 35' long (Sealand)

(3) 	8' wide X 8' high X 40' long (ISO)

d.  The RTCH is capable of handling all three sizes of containers when equipped with corresponding top handler. The reason for this is because most military containers (milvans) and most military prefab module (field hospitals, etc.) are of the 8' X 8' X 20' configuration.

e.  Every major port in this country has high speed container equipment.  This fact further enhances the idea of using containers for handling military cargo.  The commercial shipping industry would be immediately available to assist in the load out of military cargo in an emergency situation.

f.  Delivery of supplies by container versus break-bulk offers many advantages.  Supplies for a beach assault are loaded and sealed into a container in the shipping area of a logistics base.

(1) Supplies will remain intact in the container until the container lands on the beach.

(2) Damage and pilferage are virtually eliminated.

(3) Supply foul ups are reduced.

(4) More than 45,000 lbs. of supplies can be delivered safe and sound in a single movement of the RTCH.  The weight of the container must be considered when making the lift.

g.  In addition to the protection offered by containerization, a more valuable feature is the reduced time required to move the supplies inland.  The RTCH is needed to fill in the link between the landing craft and the beach in the supply chain of containerized cargo handling.

2.	CHARACTERISTICS OF THE RTCH

a.	"RTCH" is the acronym commonly used when referring to the Rough Terrain container Handler.

b.	The 988B RTCH is manufactured by Caterpillar Tractor Company.

c.	The RTCH is a 4-wheel drive, articulated steering, diesel engine powered, Rough Terrain Container Handler.

d.	The machine is designed to handle ISO and USA (Sealand) standard, 20 ft. 35 ft, and 40 ft. Top Handler Attachments.

e.	The RTCH is capable of lifting a gross weight of 50,000 lbs, and can ford 60" (5 ft) of surf.

f.	The RTCH is used for loading, unloading, handling, and stacking containers.

3.	MAJOR COMPONENTS

a.	The RTCH can be divided into two frame assemblies:

(1)	The front mast assembly.

(2)	The rear engine frame.

b.	The front mast assembly consists of the fork mast arrangement and the front axle group.

c.	The rear engine frame consists of the:

(1)	Engine assembly.

(2)	Transmission and power train.

(3)	Fuel tank.

(4)	Hydraulic reservoir.

(5)	Rear axle group.

(6)	Rear bumper assembly.

(7)	Operator's station.

d.	Front Mast Frame

(1)	Mast Arrangement:  The mast arrangement includes:

(a)	The lift cylinder.

(b)	Tilt cylinder.

(c)	Lift chains.

(d)	Carriage arrangement.

(2)	Carriage Arrangement:  The carriage arrangement includes:

(a)	Carriage and forks.

(b)	Side shift cylinders.

(c)	Tilt cylinders.

(d)	Hydraulic/Electrical lines:  Quick disconnect hydraulic and electrical fittings permit easy removal of the attachment.

(3)	The RTCH can be fitted with 20 ft, 35 ft, and 40 ft. top handler attachments.

(a) 	The 20 ft. RTCH top handler attachment, it has one cylinder.

(b) 	The 35 ft and 40 ft attachments have two cylinders, one at each end of the attachment.

e.	Rear Engine Frame

(1)	The Engine Assembly is a direct injection 65 degree V-8 Turbo Diesel engine, Model 3408T, manufactured by Caterpillar.  Max output of 393 hp at 2100 rpm.

(2) 	The RTCH has a 4-speed power shift transmission.

(3) 	The rear axle of the RTCH can oscillate plus or minus 13 degrees.  Vertical movement at the rear wheels is limited by stops to eleven inches above and below the horizontal.

(4) 	The capacity of the fuel tank, located on the left side of the RTCH, is 165 gal.  The filler opening is on top of the tank and is accessible on the machines walkway.

(5) 	The fuel system for the RTCH is equipped with a priming pump.  The pump is used to prime the fuel system on initial start up, or after changing fuel filters.  When operated, fuel is pumped from the fuel tank through the entire fuel system and back to the tank, forcing air in the filters and lines out of the system.

(6) 	The hydraulic reservoir supplies the hydraulic system with 10 wt. oil.

(a) 	The reservoir has a capacity of 78 gal.

(b)	A sight gauge installed on the side of the reservoir is used to check oil level.

(8)	Rear Bumper

(a)	A box on each side of the rear bumper in front of the counterweights, holds four 12-volt batteries each.

1  The batteries are used in the 24-volt starting, lighting and warning system.

2  The machine is capable of starting at temperatures as low as - 25 degrees F.

(b)	A storage area is located below each battery box for tools, etc.

(c)	There are steps and grab irons on both sides of the machine for access to the top of the bumper.  The top of the bumper and battery boxes are used as a platform for engine maintenance and to check the coolant level in the radiator.

(d)	The towing pintle hook is also connected to the rear bumper, and is used for towing other vehicles.

(e)	The towing lugs are used when the RTCH itself needs to be towed.

f. 	The operator's station (cab, controls, platform and seat) is a separate unit.  It can be assembled and tested as a unit.

(1)	The catwalk around the cab goes from the left side of the cab, around the back, and to the right side of the cab.  This slide also shows the hand rails which protects the operator.

(2)	Directly behind the cab of the RTCH is the primary service areas.  There are two floor panels that give access to the transmission dipstick and filler tube.

(3) 	Two doors in front of the engine open towards the cab for access to maintenance service areas.

(a)	Engine oil dipstick and filler tube.

(b)	Fuel priming pump and primary fuel filter.

(c)	Transmission oil filter.

(d)	Brake filter and air cleaner.

(4)   This slide shows the panel on the right side of the catwalk covering the hydraulic reservoir.  A valve on top of the reservoir gives protection from excessive pressure above 15 PSI.

g. 	Operators Cab

(1)	The RTCH has articulated steering.

(a) 	The steering column can be adjusted to several positions.

(b) 	When in the park position, the transmission selector lever is locked in neutral.

(c) 	Column release and park brake buttons must be pulled to operate machine.

(d)	Because the RTCH has articulating type steering, an anti-pivot link is provided for stabilization during shipping.  This pin must be removed prior to towing the RTCH.

(2)	There are three foot control levers.

(a)	The accelerator pedal controls the engine rpm.

(b)	The right brake pedal is used for normal braking.

(c)	The left brake pedal stops the machine and also neutralizes the transmission.  This allows the machine to approach a container, brake, and lift while maintaining a high engine rpm for full hydraulic power.

(3)	The horn button is located on the floor to the left of the brake pedals.

(4)	There are three instrument panels in the cab.

(a)	The left panel is a warning light panel.

(b)	The right panel is a gauge panel.

(c)	The container lock indicator panel to the right of the gauge panel, is mounted on the floor heater.  It is also a light panel that confirms the status of the lock pins for the top handler.

(5)	The control levers to the right of the operator's seat control the mast and top handler.

(6)	There are two heaters in the cab.

(a)   One in the ceiling.

(b)   One on the floor to the right of the operator.

4.	EMPLOYMENT

a.	Primarily, the RTCH is designed for employment in beach-type 
operations.

(1)   Containers stowed aboard MPS Ships are unloaded by ship-board cranes on to various types of landing craft to be transported to shore.

(2)   The RTCH is normally used to unload barge-type vessels.

(3)   The RTCH is driven onto the beached barge ferry, the container is loaded utilizing the top handler attachment, and backed to shore.

(4)	The container can be loaded by the RTCH onto flatbed trailers.

b.	The RTCH may also be used in the container staging area.

(1)   As trucks arrive loaded with containers, they are quickly unloaded and staged by the RTCH.

c.	The RTCH is capable of stacking containers two containers high to save space in the container marshaling area.